Software Architectural Design Requirements

# Introduction

A small bot on wheels needs to be designed. It should be able to move itself in an environment containing obstacles.

The obstacles can be detected and avoided thanks to an ultrasound sensor placed on a servomotor (sweeping).

Holes must be detected too with an infrared sensor (to prevent falls).

This document lists all the requirements, for the software architectural design.

Every requirement is composed of:

- One unique ID following this pattern: HLR_XXXXX (Five digits),

- A name, which is always a small introduction of the requirement,

- A text, describing what is this requirement for.

# Software Architectural Design Requirements

HLR_00100

Name: Moving forward

Text: The bot shall move forward.

Covers: SYS_00100

Module: Bot

HLR_00200

Name: Detect obstacle

Text: When the ultrasound sensor returns a value under 5cm, an obstacle can be considered detected.

Covers: SYS_00200

Module: Capteur_ultrason

HLR_00210

Name: Sweep obstacle sensor

Text: The servomotor shall rotate in a range of [-45°; +45°] in order to have an obstacle scan of 90 degrees in front of the bot

Covers: SYS_00200

Module: Servomoteur

HLR_00300

Name: Detect hole

Text: When the infrared sensor return a value over 4cm, a hole can be considered detected.

Covers: SYS_00300

Module: Capteur_infra

HLR_00600

Name: Bot start-up

Text: When the start-up command is send from the computer to the MSP430G2553 the bot should start to move in its environment.

Covers: SYS_00600

Module: Main_2553

HLR_00610

Name: Bot stop

Text: When the stop command is send from the computer to the MSP430G2553 the bot should stop moving and wait for next command to be read.

Covers: SYS_00600

Module: Main_2553

HLR_00700

Name: Data display

Text:  When the display command is sent from the user, the computer shall display sensors data information

Covers: SYS_00700

Module: Display

HLR_00800

Name: Make a 90 degree turn

Text: When the display command is sent from the computer to the MSP430G2553, the bot shall make a 90 degree turn

Covers: SYS_00700

Module: Main_2553

HLR_00810

Name: Make a 180 degree turn

Text: When the display command is sent from the computer to the MSP430G2553, the bot shall make a 180 degree turn

Covers: SYS_00700

Module: Main_2553